STREAM: Safe Terrain Recognition & Elevation Analysis for Mars using Deep Learning

Domain: Deep Learning, Computer Vision

## 1. INTRODUCTION

The exploration of Mars has entered a "New Space" era, with multiple international agencies (NASA, ESA, ISRO, CNSA) planning high-cadence missions ranging from rover deployments to eventual human habitation. A fundamental bottleneck in these missions is the "Entry, Descent, and Landing" (EDL) phase. Selecting a landing site is a high-stakes engineering challenge; hazards such as steep slopes (>15), high surface roughness, and dense boulder fields can result in the total loss of a multi-billion dollar mission.

Traditionally, this selection process is conducted by teams of planetary scientists who manually annotate thousands of high-resolution images. However, as mission frequency increases, manual analysis becomes unsustainable. The STREAM (Safe Terrain Recognition & Elevation Analysis for Mars) project proposes an automated, high-precision system leveraging state-of-the-art Deep Learning (DL) to process multimodal satellite data. By fusing 2D optical imagery with 3D elevation models, the system seeks to provide a robust, real-time safety assessment of the Martian surface.

## 2. PROBLEM STATEMENT

Current methods for landing site selection face three major challenges:

Multimodal Complexity: Difficulty in correlating 2D visual data (images) with 3D physical data (elevation).

Hazard Detection at Scale: Manual identification of craters and boulders over vast regions like the Elysium Planitia is inefficient.

Real-time Feasibility: Lack of automated pipelines that can process raw satellite tiles into actionable hazard maps for mission planning.

## 3. OBJECTIVES

The primary objectives of this project are:

- To develop a Deep Learning pipeline for semantic segmentation of the Martian surface.

- To fuse multimodal data: CTX/HiRISE optical images and MOLA/HRSC elevation data.

- To automate "Ground Truth" label generation using physics-based rules (Slope ≤ 7 degree, Low Roughness).

- To evaluate the performance of UNet/DeepLabv3+ architectures in identifying safe landing spots.

- To produce a visual heatmap overlay for easy interpretation by mission scientists.

## 4. LITERATURE REVIEW

Recent research in planetary science has shifted from traditional image processing (Hough Transforms for crater detection) to Deep Convolutional Neural Networks (CNNs).

- CNNs in Space: Studies have shown that architectures like UNet are highly effective for "pixel-level" classification of terrain.

- Multimodal Fusion: Research indicates that using elevation data (DEM) alongside RGB images significantly reduces "false positives" caused by shadows that look like craters.

- Mars Context: Existing missions (like InSight and Perseverance) have provided high-resolution datasets that serve as the gold standard for training modern ML models.

## 5. DATASET & STUDY AREA

Study Area: Landing sites of perseverance and insight for quick verification. This region is chosen due to its volcanic plains and its history as a successful landing site for NASA’s Insight mission.

Data Sources:

- MOLA (Mars Orbiter Laser Altimeter): Provides the elevation backbone (~460m/pixel).

- CTX (Context Camera): Provides medium-resolution imagery (~6m/pixel) for detecting surface texture and craters.

- Labelling Logic: Labels are generated automatically where:

  - Safe (1): Slope < 7° AND Roughness < Threshold AND No detected craters.

  - Hazardous (0): Any pixel failing the above criteria.

## 6. PROPOSED METHODOLOGY

The project follows a five-stage pipeline:

- Data Acquisition: Fetching GeoTIFF tiles from NASA PDS (Planetary Data System).

- Preprocessing: Aligning CTX images with MOLA DEM. Computation of Slope and Roughness maps using the Rasterio and GDAL libraries.

- Data Augmentation: Creating patches of $256 times 256$ pixels with rotation and flipping to improve model generalization.

- Model Training: Implementing a UNet Architecture in PyTorch. The model takes a 4-channel input: [Image, Elevation, Slope, Roughness].

- Inference & Visualization: Generating a binary mask of the test area and overlaying it on the original image to show "Green Zones" (Safe) and "Red Zones" (Hazard).

## 7. TECHNOLOGY STACK

- Programming Language: Python 3.12, Jupiter Notebook, Flask

- Frameworks: PyTorch / TensorFlow (Keras)

- Libraries: OpenCV (Image processing), Rasterio/GDAL (Satellite data), NumPy, Matplotlib.

- Environment: Google Colab with Tesla T4 GPU acceleration.VS code

## 8. EXPECTED OUTCOMES

- A trained Deep Learning model capable of segmenting Mars surface hazards with >85% IoU (Intersection over Union).

- A reusable Python pipeline for downloading and processing NASA Mars data.

- Comparative analysis between different CNN architectures for planetary terrain.

- A final project report and a research-oriented presentation.

## 9. PROJECT PLAN (GANTT CHART SUMMARY)

- Month 1: Literature Review & Environment Setup.

- Month 2: Data Acquisition & Preprocessing (Elysium Planitia).

- Month 3: Label Generation & Model Development (UNet).

- Month 4: Training, Testing, and Performance Tuning.

- Month 5: Result Analysis, Heatmap Generation & Final Documentation.

10. REFERENCES

- NASA Planetary Data System (PDS) - Geosciences Node.

- Ranneberger et al., "U-Net: Convolutional Networks for Biomedical Image Segmentation."

- Smith, D. E., et al. "Mars Orbiter Laser Altimeter: Experiment summary after the first year of global mapping of Mars."